Investigating the Effect of the Power Path Parasitic Inductance on Si-IGBT Crosstalk Using a Comprehensive Model

Amir Azam Rajabian 
Electrical Engineering Department
K. N. Toosi University of Technology
Tehran, Iran
amir.rajabian@email.kntu.ac.ir

Sadegh Mohsenzade
Electrical Engineering Department
K. N. Toosi University of Technology
Tehran, Iran
s.mohsenzade@kntu.ac.ir

Abstract—Crosstalk is an important issue in phase leg configuration of power insulated gate bipolar transistors (IGBTs). The shoot-through currents due to crosstalk increase the power loss of the devices or in dangerous conditions can break the devices. Accordingly, a comprehensive model of the crosstalk in the design phase is essential to prevent the consequences of crosstalk in practice. The present models in literature do not concentrate on the power path elements precisely. In addition, they do not involve the high-side device switching speed. This paper provides a detailed model for the crosstalk which considers the power path elements such as the path parasitic inductance. The results achieved by this model indicate an optimum band for the power path inductance which minimizes the effect of the crosstalk. This parasitic inductance can be managed in the design phase by the accurate PCB length. In addition, the effect of the DC-bus voltage on the crosstalk is assessed using the proposed model. The considered device for the analysis is IKQ50N120CH3 IGBT.

Keywords—Reliability, IGBT, Short circuit, crosstalk.

# Introduction

Power IGBTs are one of the most important components in medium/high power converters. They are widely served in traction systems [1]-[2], pulsed power supplies [3]-[4]-[5], HVDC circuit breakers [6]-[7]-[8], and solid-state transformer [9]-[10]. The reliability of such applications and uninterrupted operation are of considerable importance. According to the recent researches, 34 % of failures in power converters are related to IGBTs [11]. Hence, many researchers devoted efforts to improve IGBTs reliability in normal and harsh conditions.

Crosstalk or parasitic turn-on is a serious challenge for the IGBTs when they are used in a phase-leg structure as shown in Fig. 1. The phase-leg exists in many power converter topologies such as single/multi-phase inverters [12] and DC/DC resonant converters [13]. In a crosstalk, the turning on of the upper device parasitically turns the lower side device. Consequently, for a period of the time both devices are conducting and the shoot-through current flows among the phase-leg. More accurately, the upper device is in a complete on-state and the lower device operates in the active region during this short circuit condition. The origin of crosstalk is the voltage change of the Miller capacitance of the low-side device in transients of the high-side device form the off-state to the on-state and vice-versa. Fig. 2 presents the gate-emitter voltage of the low-side IGBT while the high-side device transients. Accordingly, when the high-side device changes state from the off to on, a positive voltage change occurs for the low side device gate-emitter voltage and a negative voltage in the turning-off process of the high-side device. When the peak value of the gate-emitter voltage

Fig. 1. The schematic of a phase leg .

Fig. 2. The gate-emitter voltage of the lower side device in an upper device transient.

(VGE1-CT in Fig. 2) exceeds the device gate threshold voltage (VTH), it can operate in the active region and saturated current can flow through the IGBTs in the leg. The extra power loss due to crosstalk can increase the devices junction temperature.

To mitigate crosstalk and its negative effects, several solutions have been proposed in the literature. Using a negative bias is proposed in [14] to decrease the gate-emitter peak voltage. This method is promising but the cost added to the system for isolated DC power supply is not acceptable in many applications. Moreover, this method cannot be served in boot-strap gate drive systems. Gate impedance regulation (GIR) is investigated in [15], [16], and [17]. The aim of these works is to optimize the gate impedance to minimize the effect of Miller capacitance on the gate side in the crosstalk. These solutions are practical but the improvement is limited to the cases with specific conditions. Several novel gate drive systems also are proposed in [18], [19], and [20]. The focus of the mentioned solutions is on the gate side. The effect of the power path on the crosstalk has not been deeply investigated yet.

This paper provides a comprehensive model for the crosstalk which considers the power path elements. More importantly, the speed of the high-side device is considered in the proposed model.  It will be revealed that the power path elements have a dominant effect on the crosstalk.  The results show a considerable dependency of the crosstalk of power path parasitic inductance. An optimum band for the power path parasitic inductance is obtained to minimize the effect of the crosstalk. This parasitic inductance depends on the PCB track length and ESL of the DC-link capacitors. Hence, in the design phase, the practitioner has a practical guideline to design the power circuit for minimum crosstalk.   In addition, the effect of the DC-bus voltage on the crosstalk is investigated. The considered device is IKQ50N120CH3 a 1200 V and 50 A discrete IGBT.

# Modelling of crosstalk

## Existing Crosstalk Models

In this section, crosstalk models are investigated. Extant  models in the literatures cannot consider the effect of important elements such as power path elements. Equation (1) is provided in [21] to inverstigate the crosstalk as

|  | VGE1=RGCGCdVCEdt1-e-tRGCGC+CGE | (1) |
| --- | --- | --- |

Expression (1) shows the effect of internal gate resistance of gate driver IC, as well as the low-side switch parasitic capacitances. The most significant capacitor in Si-IGBTs is Miller capacitance (CGC) which can determine the positive peak voltage of the spike (VGE1-CT) when the high-side IGBT turns on in the phase leg. In addition, the effect of turn-on dV/dt is applied. This model is simple and easy to analyze, however the effect of parasitic inductances and resistances are ignored.

The second and more detailed model which was discussed in [22] considers more parasitic elemnts. But, the effect of the high-side switch is ignored. The high-side device rise time plays the main role on VGE1-CT , therefore, it should be involved it in model. This circuit model applies the DC link voltage as a step function and ignores the high side turn-on device, hence the output result cannot be accurate very much. This circuit considers the parasitic inductances (LCircuit, LG, and LE), Miller capacitance (CGC) and gate resistance (RG). The results achievd by the model presented in Fig. 3 are more similar to practical results.

Fig. 3. The circuit model proposed in [21].

## Proposed Circuit Model

This paper improves the model presented in Fig. 3 in order to demonstrate the high side turn-on dV/dt, even though the body of the model still unchanged. Accordingly, the high-side device is not considered to be ideal anymore. Thus, the equations are more complex but the analysis is more accurate.

Instead of VCC in Fig. 3 and the ideal switch, Vpulse comes to the circuit in order to show the speed of high-side Si-IGBT. Fig. 4 shows how Vpulse generated theoretically so as to apply the speed of high-side switch to turn on. First, the positive ramp applies to system, then at trise the negative ramp and step function make the input voltage constant, accordingly the constant voltage is equal to Vbus. trise is the speed of high-side device which depends on the gate resistance (RG2 in Fig. 1) and the device input capacitance.

Fig. 4. Structure of Vpulse in order to apply it to the comprehensive model of crosstalk.

Serving the equivalent model of the high-side transient time is helpful to evaluate the effect of the parasitic inductances more accurately. Fig. 5 presents the proposed model for analyzing of the crosstalk.

Fig. 5. The proposed circuit model for the crosstalk analysis.

To find the gate-emitter voltage (VGE1), the Kirchhoff current law (KCL) in collector node is written. Equations (2), (3) and (4) provides the KCLs in the circuit presented in Fig. 5.

|  |  | VGRG+sLG+VG-VEsCGE+VG-VCsCGC=0 |  | (2) |
| --- | --- | --- | --- | --- |
|  |  | VE-VGsCGE+VEsLE+VE-VCsCCE=0 | (3) |
|  |  | VC-VGsCGC+VC-VEsCCE+VC-VpulseRCircuit+sLCircuit=0 |  |  |  |  | (4) |
|  |  | Vpulset=Vbustrisetut-ut-trise+Vbusut-trise |  |  |  |  |  | (5) |

Equation (5) stands for Vpulse(t) in Fig. 5 in time domain. The Laplace form of Equation (5) can be considered as Vpulse(s) and written as

|  | Vpulses=Vbus×1-e-strises2×trise | (6) |
| --- | --- | --- |

Solving the aforementioned equations gives the gate- emitter voltage of the low-side device as

|  | VGE1=CGCVBuse-s×trise×es×trise - 1×RG+LGsstrise×(D4s4+D3s3+D2s2+D1s+ 1) | (7) |
| --- | --- | --- |

where denominators are given by

| D4=CCECGCLCircuitLG+CCECGELCircuitLG+CGECGELCircuitLG+CCECGELCircuitLE+CGCCGELCircuitLE+CGCCGELGLE D3=CCECGCLCircuitRG+CCECGCLGRCircuit+CCECGELCircuitRg+CCECGELGRCircuit+CGCCGELCircuitRG+CGCCGELGRCircuit+CCECGELERCircuit+CGCCGELERCircuit+CGCCGELERG D2=CCECGCRCircuitRG+CCECGERCircuitRG+CGCCGERCircuitRG+CCELCircuit+CGCLCircuit+CGCLG+CGELG+CGELE D1=RCircuitCCE+CGC+RG(CGC+CGE) | (8) |
| --- | --- |

Expression (7) is the output of the proposed comprehensive model for the crosstalk analysis. As a result, the effect of power path elements can be analyzed analytically to find possible optimum values for the important parameters affecting the crosstalk. To check the accuracy of the proposed meodel, a case is simulated in PSPICE and and also is evaluated by the proposed model. The comparison reults is depicted in

# Crosstalk Energy

In a parasitic turn on, the high-side device is in the on-state, and the low-side device is turned on due to the voltage change of the Miller capacitcance. When both devices in a phase-leg turn on, a high level of current (IShC) flows from DC-bus to power switches (as depicted in Fig. 6) that can annihilate the Si-IGBTs. Hence, studying the energy of shoot-through current is indispensable.

The IGBT current in the saturated region can be written as [23]

| IShC=11-αPNPμnsCoxZ2LCH(VGE-VTH)2; αPNP=βPNP1+βPNP | (9) |
| --- | --- |

where μns is the average electron mobility in the channel, Cox is oxide capacitance per unit area, Z is the channel width, LCH is the channel length, and VGE is the gate-emitter voltage level. The expression (9) can be linearized and rewritten as

Fig. 6. The reprentation of the shoot-through current in a crosstalk.

| IShC=gfs(VGE1-CT-VTH); | (10) |
| --- | --- |

where gfs is the device transconductance and can be expressed by

| gfs=∂IShC∂VGE=11-αPNPμnsCoxZLCH(VGE-VTH); | (11) |
| --- | --- |

Transconductance (gfs) has a specific value in the device datasheet. Moreover it can be extracted by the slope of the curve of IGBT collector current versus gate-emitter voltage. Fig. 7 shows a typical curve of IC-VGE  to extract transconductance from INFINEON application note.

Fig. 7. The IGBTs collector current in the saturated region based on the gate-emitter voltage .

Using equation (11), the shoot-through current can be calculated based on the device parameters and VGE1-CT which can be obtained from (7). Then, the crosstalk energy (Ecrosstalk) can be calculated by

| Ecrosstalk=Vbus×IShC×tShC | (12) |
| --- | --- |

where tShC is the time in which the device gate-emitter voltage is more than VTH. This time interval can be calculated using (7).

Employing the proposed crosstalk model and extracted equations, the crosstalk energy and VGE1-CT can be calculated. Now, the effect of the power path elements on crosstalk can be evaluated.

# Effect of the power path parasitic inductance on the Crosstalk

In this section, the minimum parasitic inductance will be figured out for specific device IKQ50N120CH3 from INFINEON company with the 40 A current flow in phase leg.

Table I.	MODEL PARAMETERS AND VALUES

| CGC | ≈150 pF |
| --- | --- |
| CGE | ≈2.2 nF |
| CCE | ≈100 pF |
| LE | ≈7-13 nH |
| LG | ≈1-5 nH |
| RCircuit | ≈1 Ω |
| Vbus | 600 V |

To find the inductance of traces in PCBs, there is a formula that gives the approximate parasitic inductance, but first of all, the trace width should be figured out. Trace width are determined with the IPC-2221 standard thus with the given values, there is a specific inductance.

To find out the PCB width which is needed for selected values, there is an equation that gives the track width. First, the area of PCB track should be found out. Equation (10) is gives the area, thus Itr is the current which flows from trace, Tr is the rising temperature of the PCB and k, b and c are constants which are assigned from IPC-2221 standard.

|  | A=ItrkTrb1c | (13) |
| --- | --- | --- |

Moreover, after finding out the area of trace, equation (13) provides the trace width by using the given parameters, hence the thickness (T) of the track is needed.

|  | W=AT×1.378 | (14) |
| --- | --- | --- |

Hence, the width of PCB trace is given. parameters and their value in ambient temperature of 25°C is given in Table 2.

Table II.	MODEL PARAMETERS AND VALUES

| k | 0.048 |
| --- | --- |
| b | 0.44 |
| c | 0.725 |
| T (Thickness) | 1 oz/ft2 |
| Tr (Temperature rise) | 40 °C |
| Itr | 40 A |
| L (trace Lenght) | 7 cm |

Next step is calculating the inductance of the path. The formula to figure out the inductance of PCB is from [24]. Equation (12) provides a formula according to substrate height (H), trace width (W), trace thikness (T) and trace length (L), moreover two simplify the formula, assume that trace width≫substrate heigt≫trace thickness.

|  | L≈μ0μr×H+T2×LW | (15) |
| --- | --- | --- |

Hence, the minimum inductance of the path is found out, in addition of this, اندوکتانس 50 نانویی.

Next step is figuring out the dependence of crosstalk energy and gate-emitter voltage on parasitic inductance of power path. Anlysis is based on the model in Fig. 4 of this paper. As mentioned, the minimum inductance of the path is 60 nH, hence the starting point of analysis is LCircuit equal to 60 nH. There is no limitation for maximum point otherwise the maximum point is 240 nH wisely.

The values of gate resistance and rise time of high side switch is selected from IKQ50N120CH3 device datasheet.

Figure (6) shows the peak gate-emitter voltage of the spike during shoot-through according to parasitic inductance (LCircuit).

Fig. 8. Effect of parasitic inductance on gate-emitter voltage.

Hence, all parameters to figure out of crosstalk energy is given. The duration of this phenomenon is calculated by MATLAB and SPICE, furthermore the maximum voltage of gate-emitter gives a good perspective that what periods must be used in order to avoid shoot-through phenomenon.

Using equations (10) and (12) provides the crosstalk energy according to parasitic inductance of power path. Due to values of gate-emitter peak voltages finding out the crosstalk energy is simple and can be analysed accurately. Fig. 7 shows the shoot-through energy according to LCircuit.

Fig. 9. Crosstalk energy in SPICE and MATLAB

The results show that that by choosing an appropriate trace length for pathes in a way that parasitic inductance goes to safe zone in order to minimal shoot-through energy and current. The safe area for parasitic inductance is 160 nH to 200 nH.

# Effect of the transient speed of the high-side device on the crosstalk

In this section, the effect of DC-bus voltage is investigated, moreover the best inductance of the power path is selected so it is used in this part.

The effect of high side device depends on the gate resistance, hence by choosing a proper value for gate resistance, the transient speed of high side switch can be controlled. In this paper, for evaluating the effect of parasitic inductance, the value of gate resistor is chosen from IKQ50N120CH3 device datasheet, moreover the rise time is given too. The rate of increasing or decreasing of rise time of high-side device is modified linearly, thus by choosing a proper inductance, such as 180 nH, the effect of high-side transient speed can be analysed. Fig. (8) shows the effect of gate resistance and transient speed on gate-emitter spike voltage.

Fig. 8 shows the peak voltage of gate-emitter under various gate resistances.

Fig. 10. Effect of transient speed of high-side device.

# Effect of the DC-Bus voltage on the crosstalk

# Referances

[1]	S. Dieckerhoff, S. Bernet, and D. Krug, “Power loss-oriented evaluation of high voltage IGBTs and multilevel converters in transformerless traction applications,” IEEE Transactions on Power Electronics, vol. 20, no. 6, pp. 1328–1336, Nov. 2005, doi: 10.1109/TPEL.2005.857534.

[2]	Z. Xu, M. Li, F. Wang, and Z. Liang, “Investigation of Si IGBT Operation at 200$\,^\circ$ C for Traction Applications,” IEEE Transactions on Power Electronics, vol. 28, no. 5, pp. 2604–2615, May 2013, doi: 10.1109/TPEL.2012.2217398.

[3]	S. Mohsenzade, M. Zarghani, and S. Kaboli, “A High-Voltage Series-Stacked IGBT Switch With Active Energy Recovery Feature for Pulsed Power Applications,” IEEE Transactions on Industrial Electronics, vol. 67, no. 5, pp. 3650–3661, May 2020, doi: 10.1109/TIE.2019.2921297.

[4]	M. Zarghani, S. Mohsenzade, and S. Kaboli, “A Series Stacked IGBT Switch Based on a Concentrated Clamp Mode Snubber for Pulsed Power Applications,” IEEE Transactions on Power Electronics, vol. 34, no. 10, pp. 9573–9584, Oct. 2019, doi: 10.1109/TPEL.2019.2894994.

[5]	J. N. Nasab, A. Hadizade, S. Mohsenzade, M. Zarghany, and S. Kaboli, “A Marx-based generator with adjustable FWHM using a controllable magnetic switch,” IEEE Transactions on Dielectrics and Electrical Insulation, vol. 26, no. 2, pp. 324–331, Apr. 2019, doi: 10.1109/TDEI.2018.007522.

[6]	S. Mohsenzade, M. Zarghani, and S. Kaboli, “A Series Stacked IGBT Switch to Be Used as a Fault Current Limiter in HV High Power Supplies,” IEEE Journal of Emerging and Selected Topics in Power Electronics, pp. 1–1, 2021, doi: 10.1109/JESTPE.2021.3061093.

[7]	S. Mohsenzade, M. Zarghani, and S. Kaboli, “A Voltage Balancing Method for Series-Connected IGBTs Operating as a Fault Current Limiter in High-Voltage DC Power Supplies,” IEEE Transactions on Industrial Electronics, vol. 68, no. 9, pp. 7895–7907, Sep. 2021, doi: 10.1109/TIE.2020.3009606.

[8]	E. Deng et al., “Research on the Multi-Physics Field-Circuit Coupling Model of Press Pack IGBT Considering the Application of Hybrid HVDC Breakers,” IEEE Journal of Emerging and Selected Topics in Power Electronics, pp. 1–1, 2020, doi: 10.1109/JESTPE.2020.3019433.

[9]	J. Zhang, J. Liu, S. Zhong, J. Yang, N. Zhao, and T. Q. Zheng, “A Power Electronic Traction Transformer Configuration With Low-Voltage IGBTs for Onboard Traction Application,” IEEE Transactions on Power Electronics, vol. 34, no. 9, pp. 8453–8467, Sep. 2019, doi: 10.1109/TPEL.2018.2889107.

[10]	X. Zhao et al., “DC Solid State Transformer Based on Three-Level Power Module for Interconnecting MV and LV DC Distribution Systems,” IEEE Transactions on Power Electronics, vol. 36, no. 2, pp. 1563–1577, Feb. 2021, doi: 10.1109/TPEL.2020.3007674.

[11]	U.-M. Choi, F. Blaabjerg, and K.-B. Lee, “Study and Handling Methods of Power IGBT Module Failures in Power Electronic Converter Systems,” IEEE Transactions on Power Electronics, vol. 30, no. 5, pp. 2517–2533, May 2015, doi: 10.1109/TPEL.2014.2373390.

[12]	M. Zarghani, S. Mohsenzade, A. Hadizade, and S. Kaboli, “An Extremely Low Ripple High Voltage Power Supply for Pulsed Current Applications,” IEEE Transactions on Power Electronics, pp. 1–1, 2020, doi: 10.1109/TPEL.2020.2966682.

[13]	M. Zarghani, S. Mohsenzade, and S. Kaboli, “A Very Low-Ripple High-Voltage High-Power DC Power Supply Using an Interleaved Converter and a Linear Ripple Elimination Unit,” IEEE Journal of Emerging and Selected Topics in Power Electronics, vol. 9, no. 3, pp. 3339–3352, Jun. 2021, doi: 10.1109/JESTPE.2020.3040072.

[14]	W. W. T. Chan, K. O. Sin, P. K. T. Mok, and S. S. Wong, “A power IC technology with excellent cross-talk isolation,” IEEE Electron Device Letters, vol. 17, no. 10, pp. 467–469, Oct. 1996, doi: 10.1109/55.537077.

[15]	Z. Zhang, F. Wang, L. M. Tolbert, and B. J. Blalock, “Active Gate Driver for Crosstalk Suppression of SiC Devices in a Phase-Leg Configuration,” IEEE Transactions on Power Electronics, vol. 29, no. 4, pp. 1986–1997, Apr. 2014, doi: 10.1109/TPEL.2013.2268058.

[16]	Y. Li, M. Liang, J. Chen, T. Q. Zheng, and H. Guo, “A Low Gate Turn-OFF Impedance Driver for Suppressing Crosstalk of SiC MOSFET Based on Different Discrete Packages,” IEEE Journal of Emerging and Selected Topics in Power Electronics, vol. 7, no. 1, pp. 353–365, Mar. 2019, doi: 10.1109/JESTPE.2018.2877968.

[17]	H. Li, Y. Jiang, Z. Qiu, Y. Wang, and Y. Ding, “A Predictive Algorithm for Crosstalk Peaks of SiC MOSFET by Considering the Nonlinearity of Gate-Drain Capacitance,” IEEE Transactions on Power Electronics, vol. 36, no. 3, pp. 2823–2834, Mar. 2021, doi: 10.1109/TPEL.2020.3016155.

[18]	B. Zhang, S. Xie, J. Xu, Q. Qian, Z. Zhang, and K. Xu, “A Magnetic Coupling Based Gate Driver for Crosstalk Suppression of SiC MOSFETs,” IEEE Transactions on Industrial Electronics, vol. 64, no. 11, pp. 9052–9063, Nov. 2017, doi: 10.1109/TIE.2017.2736500.

[19]	M. Shen, “A simple gate assist circuit for SiC devices,” in 2014 IEEE Conference and Expo Transportation Electrification Asia-Pacific (ITEC Asia-Pacific), Aug. 2014, pp. 1–6. doi: 10.1109/ITEC-AP.2014.6941017.

[20]	Z. Zhang, J. Dix, F. F. Wang, B. J. Blalock, D. Costinett, and L. M. Tolbert, “Intelligent Gate Drive for Fast Switching and Crosstalk Suppression of SiC Devices,” IEEE Transactions on Power Electronics, vol. 32, no. 12, pp. 9319–9332, Dec. 2017, doi: 10.1109/TPEL.2017.2655496.

[21]	Z. Zhang, W. Zhang, F. Wang, L. M. Tolbert, and B. J. Blalock, “Analysis of the switching speed limitation of wide band-gap devices in a phase-leg configuration,” in 2012 IEEE Energy Conversion Congress and Exposition (ECCE), Sep. 2012, pp. 3950–3955. doi: 10.1109/ECCE.2012.6342164.

[22]	S. Jahdi, O. Alatise, J. A. Ortiz Gonzalez, R. Bonyadi, L. Ran, and P. Mawby, “Temperature and Switching Rate Dependence of Crosstalk in Si-IGBT and SiC Power Modules,” IEEE Transactions on Industrial Electronics, vol. 63, no. 2, pp. 849–863, Feb. 2016, doi: 10.1109/TIE.2015.2491880.

[23]	S. Mohsenzade, M. Zarghany, and S. Kaboli, “A Series Stacked IGBT Switch With Robustness Against Short-Circuit Fault for Pulsed Power Applications,” IEEE Transactions on Power Electronics, vol. 33, no. 5, pp. 3779–3790, May 2018, doi: 10.1109/TPEL.2017.2712705.

[24]	T. H. Hubing, T. P. Van Doren, and J. L. Drewniak, “Identifying and quantifying printed circuit board inductance,” in Proceedings of IEEE Symposium on Electromagnetic Compatibility, Aug. 1994, pp. 205–208. doi: 10.1109/ISEMC.1994.385661.